M4H1. Definition de Architecture

11/08/2026

Eugenio Loeza-A01067846

Diego Ibarra-A01644350

Diego Angulo-A01643797

Gael Castillo-A01638638

Project Planning

1. Objective

Measure noise pollution on three floors of a college building and log all readings into a single time-stamped CSV.

2. System Architecture

Three ESP32 nodes, one per floor, each with a KY-038 analog sound sensor.

Nodes send their readings over ESP-NOW to a fourth ESP32 that acts as a gateway.

The gateway connects to the Pi over USB and forwards each packet as a serial line.

The Pi parses the serial stream and appends rows to the CSV.

A gateway is needed because ESP-NOW works only between ESP devices; the Pi cannot receive those frames.

3. Hardware per Node

ESP32 development board (ESP32-WROOM-32 or equivalent), USB powered.

KY-038 sound sensor, analog output A0 to GPIO34 (ADC1_CH6), powered from 3.3 V.

Sensitivity potentiometer tuned per node during calibration.

4. Data Format

CSV columns: Timestamp, Node_ID, Info_Sens_1, Info_Sens_2, Info_Sens_3, Label.

Timestamp: assigned by the Pi on reception, ISO 8601 with milliseconds.

Node_ID: 1, 2 or 3, hard-coded in each node's firmware.

Info_Sens_1 to Info_Sens_3: three raw ADC readings (0-4095) from the same sampling window.

Label: floor identifier (floor_1, floor_2, floor_3), derived from Node_ID by the logger.

5. Firmware Plan

Nodes: register the gateway MAC as an ESP-NOW peer at startup.

Nodes: once per second, take three ADC readings and send them with the Node_ID in a fixed struct.

Nodes: check the ESP-NOW delivery callback and retry on failure.

Gateway: on each received packet, print one fixed-format line to serial at 115200 baud.

6. Raspberry Pi Logger

Python service that reads the serial port, validates each line, and appends to the CSV.

Writes the header if the file is new and flushes every row to survive power loss.

Runs as a systemd service so logging restarts after a reboot.

7. Testing and Deployment

Bench test all three nodes next to the gateway before installing anything.

Verify ESP-NOW range from each floor to the chosen gateway location.

Install one node per floor, record which Node_ID is where, and run a 24-hour capture.

8. Risks

Range through concrete floors may be poor; mitigate by placing the gateway in a stairwell.

The KY-038 is uncalibrated, so results are comparative between floors, not absolute decibels.

Simultaneous transmissions may overrun the serial buffer; stagger the sampling offset per node.